ENG40011 — Alzheimer's Care Device

Comprehensive Task List

End-to-end build for prototype delivery & final submission

10 workstreams · ~165 tasks · grouped for parallel execution

# How to Use This List

Every task carries a workstream prefix and a sequence number (e.g. FW-08). Subtasks under each task are scope, not separate trackable items — they tell whoever owns the task what "done" looks like. Tasks are grouped by workstream rather than by week so the team can run in parallel; refer back to the unit project plan for week-by-week milestones.

Parallel-safe: the four workstreams that can run independently from day one are Hardware, Firmware, Backend, and Dashboard UI. MQTT, Indoor Positioning, and Integration converge them.

## Workstream Summary

| Prefix | Workstream | Scope |
| --- | --- | --- |
| PM | Project Management | Coordination, milestones, repo, risk register, demo prep |
| HW | Hardware | Wearable bench-up, BLE beacons, enclosure, batteries, mounting |
| FW | Firmware | ESP32-S3 code: sensors, scanning, MQTT, power, OTA |
| MQ | MQTT Layer | Broker setup, topics, ACLs, QoS, TLS, scaling path |
| BE | Backend | Supabase schema, RLS, Edge Functions, ingestion, rules engine |
| POS | Indoor Positioning | RSSI math, fingerprinting, fusion, smoothing, mode switching |
| UI | Caregiver Dashboard | React + Shadcn SaaS: auth, patient mgmt, floor plan, alerts, history |
| TST | Integration & Testing | E2E smoke tests, latency, accuracy, reliability, UX walkthrough |
| EV | Evaluation | Baseline, metrics, false-rate analysis, Biodesign mapping |
| REG | Regulatory & Privacy | TGA, ISO/IEC, Privacy Act, GDPR, consent, retention, ethics |
| DOC | Documentation | Architecture doc, BoM, final report, deck, demo script |

PM · Project Management · 7 tasks

PM-01  Confirm workstream owners

- Lock in lead per workstream: HW, FW, BE, UI, REG, DOC

- Document who is the backup for each so no single point of failure

- Publish ownership map in repo README

PM-02  Set up shared repository & access

- GitHub org/repo with branch protection on main

- Invite all team members with appropriate permissions

- Pin issue templates and a CONTRIBUTING.md

- Connect to Vercel + Supabase for auto-deploy

PM-03  Establish weekly standup cadence

- Fixed slot, 15-30 min, async written status if anyone misses

- Standing agenda: blockers, last-week deliverables, this-week deliverables

- Single shared notes doc updated each meeting

PM-04  Maintain risk register

- Top risks identified upfront: hardware procurement, BLE accuracy, scope creep, regulatory blind spots

- Owner + mitigation per risk

- Reviewed weekly at standup

PM-05  Track milestones against unit deliverables

- Milestone 1: Assignment 2 (Wk 3 — solution concept)

- Milestone 2: Assignment 3 (Wk 5 — prototype + evaluation)

- Milestone 3: Final Report + Presentation (Wk 6)

- Each milestone has a go/no-go checkpoint 3 days prior

PM-06  Credentials & secrets sharing strategy

- Shared password manager (1Password / Bitwarden)

- Document who holds keys for Supabase, MQTT broker, Vercel, Mapbox

- Establish rotation plan if anyone leaves the project

PM-07  Demo schedule and dry runs

- Two dress rehearsals before final presentation

- Backup video recording of working demo (in case live fails)

- Roles assigned: who narrates, who drives the laptop, who handles Q&A

HW · Hardware · 20 tasks

HW-01  Finalise wearable BoM

- Confirm parts list reflects ESP32-S3-LCD-1.47B (replaces nRF52840 + IMU lines)

- Confirm MAX30102 module (DFRobot SEN0344 V2.0) replaces raw sensor in V1

- Cost roll-up per unit + total prototype build cost

HW-02  Procure wearable components

- Order ESP32-S3 dev board(s) + spares

- Order MAX30102 modules + spares

- Order JST connectors, cabling, breadboards / protoboards

- Track delivery dates against build window

HW-03  Source BLE beacons

- Minimum 4 beacons to give triangulation headroom in test space

- Confirm configurable TX power, advertising interval, and stable MAC

- Order spares (one beacon failure shouldn't stall a demo)

HW-04  Configure BLE beacons

- Set TX power to a known value across all beacons

- Set advertising interval (100-200 ms is a good starting point)

- Label each beacon physically with its short ID + log MAC addresses

HW-05  Source enclosure & wearable materials

- 3D printer access confirmed; filament ordered

- Strap or clip mechanism material chosen

- Skin-contact safe materials documented

HW-06  Bench setup for ESP32-S3

- USB drivers installed (CP2102 or native CDC)

- Arduino IDE with Espressif ESP32 core 3.x installed

- Board profile saved: ESP32S3 Dev Module, USB CDC On Boot enabled, PSRAM=OPI, Flash=16MB

HW-07  Validate onboard LCD output

- Run vendor demo to confirm 1.47" 172×320 IPS works

- Use LCD as an in-field debug surface (status, last reading)

HW-08  Wire MAX30102 module to ESP32-S3

- I²C bus 0x57 connection confirmed (SDA, SCL, VCC, GND)

- Pull-ups verified

- UART fallback noted as alternative

HW-09  Validate MAX30102 reads

- First reading after ~8s contact (per module behaviour)

- HR, SpO2, temperature returned and not -1 in steady state

- Sanity-check values against a reference pulse oximeter

HW-10  Validate onboard QMI8658 IMU

- I²C addr 0x6A or 0x6B confirmed

- Accel + gyro readings stable when stationary

- Magnitudes change correctly under known motion (drop, tilt)

HW-11  Validate BLE scanning

- ESP32-S3 detects all configured beacons within 10 metres

- RSSI values logged at known distances to seed path-loss calibration

HW-12  Validate WiFi connect & scan

- WiFi station mode connects to test network

- Scan returns AP RSSI list usable for fingerprinting

HW-13  Validate TF card logging

- Filesystem mounted, write/read confirmed

- Plan: rotating logs, max file size, automatic deletion of old files

HW-14  Battery integration & charging

- Li-ion cell sized for target runtime (3-5 day target)

- Onboard charger circuit verified (charge & discharge cycles)

- Low-battery cutoff threshold documented

HW-15  Enclosure design v1 (CAD)

- Internal volume sized for board + battery + sensor module

- Strap/clip mounting features

- Access for charging port

- Aesthetic + ergonomic review

HW-16  Print and assemble enclosure

- Test fit

- Iterate dimensions if needed

- Final assembled wearable for demo

HW-17  Wearable mounting / strap

- Comfort test on team member for 1+ hour wear

- Adjustability across wrist sizes

HW-18  Battery life measurement

- Continuous operation under realistic duty cycle

- Plot battery voltage vs time

- Update spec sheet with measured runtime

HW-19  IP rating / sealing test

- At minimum, splash resistance demonstrated

- Document level claimed (IPx2 vs IPx4 etc.)

HW-20  Beacon physical mounting in test space

- Mount at consistent heights (e.g. 2 m) and known x/y positions

- Avoid metal surfaces or RF-blocking obstructions

- Document layout in a sketch matching what will be drawn in the dashboard floor plan

FW · Firmware · 20 tasks

FW-01  Initialise firmware project

- Arduino IDE workspace committed to monorepo at /firmware

- Library dependencies pinned (DFRobot_BloodOxygen_S, PubSubClient or AsyncMqttClient, ArduinoJson, etc.)

- Build script that flashes from CLI for repeatability

FW-02  Confirm build configuration

- Espressif ESP32 core 3.x

- Board: ESP32S3 Dev Module

- USB CDC On Boot: enabled

- PSRAM: OPI

- Flash: 16MB

FW-03  Integrate MAX30102 driver

- Use DFRobot_BloodOxygen_S (NOT DFRobot_MAX30102)

- Begin/sensorStartCollect/getHeartbeatSPO2/getTemperature_C/sensorEndCollect cycle

- Handle ~8s contact warmup; flag readings as invalid until stable

- Treat -1 returns as invalid; don't propagate downstream

FW-04  IMU driver (QMI8658)

- I²C init at 0x6A/0x6B

- Configure accel + gyro ranges and ODR

- Wrapper API: getAccel(), getGyro()

FW-05  Sensor sampling loop

- Duty-cycle: vitals every N seconds, IMU at higher rate, BLE/WiFi scan periodically

- Timestamps via NTP or millis offset

- Buffer circular queue if MQTT publish backs up

FW-06  Fall detection algorithm

- Detect impact spike in accel magnitude

- Look for post-impact stillness

- Emit fall event message on `events` topic

FW-07  Activity classification stub

- Simple thresholds: still / walking / agitated

- Pluggable interface so V2 ML can drop in later

FW-08  BLE scanning task

- Periodic scan (e.g. every 1-2 s)

- Filter to known beacon MACs

- Return RSSI per beacon to publishing task

FW-09  WiFi RSSI scanning

- Periodic scan of nearby APs

- Capture BSSID + RSSI list

- Throttle to avoid hammering the radio

FW-10  WiFi connection manager

- Auto-reconnect with backoff

- Credentials provisioning (SmartConfig or BLE provisioning at first boot)

- Status surfaced to LCD

FW-11  MQTT client integration

- Connect to broker with TLS + credentials

- Last-Will-Testament message for offline detection

- Reconnect with backoff

FW-12  MQTT topic schema implementation

- device/{patient_id}/telemetry — vitals, IMU summary

- device/{patient_id}/signals — BLE/WiFi RSSI snapshots

- device/{patient_id}/events — fall, button press, low battery

FW-13  JSON message formatter

- Compact field names to save bytes

- Schema documented and matched on backend

- Versioned message format for forward compatibility

FW-14  Local TF card buffer

- If MQTT unavailable, write messages to TF card

- On reconnect, drain backlog in order

- Cap backlog size; oldest data dropped first

FW-15  Time synchronisation

- NTP sync on connect

- Fall back to monotonic timestamps if no internet

- Backend reconciles drift on ingest

FW-16  Power management

- Light sleep between scans

- Disable LCD backlight when no recent caregiver interaction

- Document measured duty-cycle current draw

FW-17  LCD status feedback

- Boot screen with device ID

- Live indicators: WiFi, MQTT, battery, last reading

- Error code display when something fails

FW-18  Firmware versioning & device ID

- Embed firmware version in every message payload

- Use chip MAC as immutable device ID

- Surface to backend on first connect (devices.firmware_version)

FW-19  Enrollment / pairing handshake

- On first connect, device announces itself

- Backend creates a pending devices row awaiting caregiver pairing

- Stretch: shake-to-pair or button-press confirmation

FW-20  OTA update mechanism (stretch)

- ArduinoOTA or ESP-IDF OTA

- Versioned firmware artefacts in cloud storage

- Rollback path if update bricks device

MQ · MQTT Layer · 9 tasks

MQ-01  Choose broker hosting strategy

- Mosquitto in Docker for local dev

- Cloud-hosted Mosquitto (Fly.io, EC2, or Railway) for prototype demo

- AWS IoT Core noted as production swap-in

MQ-02  Deploy Mosquitto in Docker

- docker-compose definition committed to repo

- Persistent volume for messages and ACL data

- Reachable from team networks for collaborative testing

MQ-03  TLS configuration

- Self-signed certs OK for prototype, but configured (not disabled)

- CA cert distributed to firmware build for verification

MQ-04  Auth setup

- Username/password file managed

- Per-device credentials, not a shared account

- Rotation procedure documented

MQ-05  Topic ACLs

- Devices can publish only to their own patient's topics

- Backend bridge has read-all access

- No client subscribes to wildcard topics in production

MQ-06  QoS strategy

- QoS 0 for high-frequency telemetry (loss tolerable)

- QoS 1 for signals snapshots

- QoS 2 for events (alerts cannot be lost)

MQ-07  Last-Will-Testament setup

- Each device sets a will message announcing offline state

- Backend listens for these and updates device.last_seen accordingly

MQ-08  Broker monitoring

- Log retention strategy

- Dashboard or query for connected device count

- Alert if broker is unreachable from backend

MQ-09  Production swap-in plan

- Document the 3-5 changes needed to point at AWS IoT Core

- Include cert handling differences

- Include cost estimate at expected scale

BE · Backend · 13 tasks

BE-01  Create Supabase project(s)

- Dev project for active build

- Prod-like project for demo

- Lock down API keys; configure environments

BE-02  Schema migration v1 (core entities)

- caregivers, patients, caregiver_patient

- devices, beacons, floor_plans, calibration_points

- Migrations checked into supabase/migrations folder

BE-03  Schema migration v2 (telemetry & alerts)

- sensor_readings, position_estimates

- alert_rules, alerts, audit_log

- Composite indexes on (patient_id, recorded_at)

BE-04  Row-Level Security policies

- Caregivers can only access their assigned patients

- Devices can write only to their own patient's data

- Audit log readable only by admins

BE-05  Realtime publications

- Enabled on sensor_readings, position_estimates, alerts

- Tested via Supabase JS client subscription

BE-06  Edge Function: mqtt_bridge

- Subscribes to MQTT broker

- Validates JSON payloads against shared Zod schemas

- Writes to sensor_readings / position-relevant tables

- Idempotent on duplicate messages

BE-07  Edge Function: position_estimator

- Triggered by new signals payloads

- Calls trilateration + fingerprint matcher

- Writes position_estimates row

- Mode flag (indoor/outdoor) set per available data

BE-08  Edge Function: rules_engine

- Triggered by new sensor_readings or position_estimates

- Loads enabled alert_rules for the patient

- Evaluates each rule type (zone, vitals, fall, inactivity)

- Writes alerts row when conditions match

- Throttles repeated alerts (cooldown window)

BE-09  Auth configuration

- Email/password and magic link enabled

- Email templates branded

- Caregiver role attribute on signup

BE-10  Storage bucket for floor plan assets

- If caregivers upload floor plan images, bucket configured

- Per-patient access control on stored files

BE-11  Audit log triggers

- Auto-log: device pairing, beacon placement, rule changes, acknowledgement actions

- Read-only for caregivers

BE-12  Seed demo data

- One demo patient, one device, four beacons placed

- Sample alert rules pre-configured

- Synthetic 24h history to populate charts

BE-13  Backup & restore plan

- Daily automated backups via Supabase

- Restore procedure documented

- Tested at least once before final demo

POS · Indoor Positioning · 10 tasks

POS-01  RSSI-to-distance path-loss model

- Choose log-distance model (n exponent + reference RSSI)

- Calibrate at known distances per beacon batch

POS-02  Per-beacon calibration constants

- Stored per beacon row (TX power, calibrated RSSI@1m)

- Updatable from dashboard if a beacon is replaced

POS-03  Trilateration solver

- Wrap Trilateration.js with our distance estimates

- Handle 3-beacon and 4+ beacon cases gracefully

- Reject solutions with high residual error

POS-04  Fingerprint matcher

- Compose BLE + WiFi RSSI vector at runtime

- k-Nearest-Neighbours against calibration_points

- Weighted average of k positions for output

POS-05  Sensor fusion (trilateration + fingerprint)

- Choose blend strategy (e.g. weighted by confidence)

- Single position output with confidence score

POS-06  Smoothing filter

- Light Kalman or moving-average over last N estimates

- Suppress jitter without introducing visible lag

POS-07  Confidence scoring

- Combine residual error + signal availability + fingerprint match quality

- Surface confidence to dashboard (e.g. dimmer marker when low)

POS-08  Mode switch indoor ↔ outdoor

- Use last-known indoor confidence + GPS validity

- Hysteresis to prevent flapping at the doorway

POS-09  GPS validity gating

- HDOP threshold to reject low-quality fixes

- Drop the first N seconds of GPS after a cold start

POS-10  Stress testing with synthetic data

- Replay engineered RSSI sequences to verify edge cases

- Cover beacon dropout, NLOS scenarios, multipath spikes

UI · Caregiver Dashboard · 30 tasks

UI-01  Repo & build scaffold

- Vite + React + TypeScript + Tailwind + Shadcn UI

- ESLint, Prettier, strict TS

- Vercel preview deployments per PR

UI-02  Brand theme tokens

- Navy / steel / cream / orange tokens wired into Tailwind

- Typography: Century italic for titles, Inter for body

UI-03  Authentication pages

- Sign up, log in, password reset

- Role selection (professional vs family) on signup

UI-04  Protected route wrapper

- Redirect unauthenticated users to login

- Session restoration on reload

UI-05  Caregiver profile page

- Edit name, contact, role

- Preferences (units °C/°F, notification channels)

UI-06  Patient roster page

- List of allocated patients with status pill + alert badge

- Search and filter

- Empty state when no patients allocated

UI-07  Patient creation flow

- Form: name, dob, notes

- Available to professional caregivers; family caregivers have a different entry path

UI-08  Patient detail dashboard shell

- Tabs: Live, Floor Plan, Map, Alerts, Settings, History

- Persistent header with patient name + status

- Patient stream subscription mounted at this level

UI-09  Live sensor cards

- HR, SpO2, skin temp, motion state

- Last-update timestamp; stale-data warning at 30s+

- Connection status indicator

UI-10  Sensor sparklines

- Last 5 min per metric

- Fixed-size buffer to avoid memory growth

UI-11  Floor plan editor

- Fabric.js canvas

- Walls (lines), rooms (polygons), doors (annotations)

- Measurement input → scale_meters_per_pixel

- Save / load to/from Supabase

UI-12  Furniture layer

- Drag-and-drop common items: bed, chair, table, toilet, kitchen

- Persists alongside floor plan

- Useful for V2 behavioural ML context

UI-13  Beacon discovery & pairing

- Surface beacons reported by the patient's wearable

- Pair flow: select, label, confirm

- Validation: at least 3 paired before triangulation enabled

UI-14  Beacon placement on canvas

- Drag beacon markers onto floor plan

- Snap-to-corner helpers

- Edit / remove flow

UI-15  Calibration walk UI

- Suggested calibration points overlaid on floor plan

- Capture button records BLE+WiFi snapshot at each point

- Progress indicator until quality threshold met

- Recalibrate option

UI-16  Live patient marker on floor plan

- Animated dot updated from position_estimates Realtime feed

- Confidence visualised (opacity or halo radius)

- Trail showing last few seconds of motion

UI-17  Outdoor map view

- Mapbox GL JS with branded styling

- Live patient pin, last-known indoor exit point, breadcrumb of last 30 min

- Auto-switch from floor plan when mode = outdoor

UI-18  Geofence drawing

- Caregiver draws polygon zones on the map

- Save as part of the patient's alert rules

- Breach triggers an alert via the rules engine

UI-19  Device pairing dialog

- Add device by MAC or pick from discovered list

- Default-pair to current patient context

- Validation when device already paired elsewhere

UI-20  Alert rule configuration

- Per-patient settings tab grouped by rule type

- Rule types: zone, vitals, fall, inactivity, repetitive movement

- Severity, channel, enable/disable

- Live preview of what would have alerted in last 24h

UI-21  Alert feed (global)

- Bell icon in header with count badge

- Dropdown listing recent alerts across all assigned patients

- Click an alert to deep-link to that patient's Alerts tab

UI-22  Alert tab (per patient)

- Full historical alert list with filtering

- Acknowledgement workflow (sets acknowledged_at + ack_by)

- Severity colour coding

UI-23  Critical alert audio + visual cue

- Browser tab audio for critical alerts

- Notification API for OS-level prompt when tab inactive

UI-24  History scrubber

- Time slider on the floor plan canvas

- Replay last N hours of patient movement at variable speed

UI-25  Vitals history charts

- Selectable date range (last hour, last day, last week)

- Recharts line charts for HR/SpO2/temp

UI-26  CSV export

- Selectable date range and metric set

- Used for caregiver handover or clinical review

UI-27  Loading, empty, and error states

- Skeleton placeholders on every panel

- Helpful empty states with next-step prompts

- Friendly error messages with retry

UI-28  Accessibility pass

- Keyboard navigation across all primary flows

- ARIA labels on canvas-based components

- Colour-contrast checks

UI-29  Mobile responsiveness

- Caregiver may check the dashboard on their phone

- Critical views collapse to single column gracefully

UI-30  Vercel deploy & preview branches

- Auto-deploy on merge to main

- PR previews enabled for design review

TST · Integration & Testing · 14 tasks

TST-01  Sensor → broker E2E smoke test

- Wearable publishes; messages observable on broker

- Schema validation passes

TST-02  Broker → Postgres E2E test

- Bridge function persists messages with correct shape

- No duplication on retry

TST-03  Postgres → dashboard Realtime test

- New rows surface in dashboard within 2s

- Subscription survives reconnect

TST-04  Full chain latency measurement

- Inject simulated event at firmware

- Measure time-to-alert in caregiver UI

- Target: under 30s end-to-end

TST-05  Indoor position accuracy test

- Walk a known path through calibrated test space

- Compare estimated trajectory to ground truth

- Target: <1.5m error on 80% of samples

TST-06  Outdoor GPS accuracy test

- Walk a known outdoor route

- Compare GPS pin to ground truth

TST-07  Fall detection rate

- Controlled drop tests of the wearable on a soft surface

- Measure true positive and false positive rates

TST-08  Geofence breach detection

- Walk across a defined geofence boundary

- Confirm alert fires within target latency

TST-09  Alert delivery reliability

- 10 trigger samples per rule type

- All deliveries land in the alert feed

- No phantom alerts

TST-10  Continuous 60-min stability run

- Wearable + broker + dashboard run uninterrupted

- No memory leaks, no client reconnect storms

TST-11  Battery life under realistic duty cycle

- Run device until cutoff voltage

- Plot voltage curve and report runtime

TST-12  Network drop / reconnect behaviour

- Disable broker mid-test; verify TF card buffering

- Re-enable; verify backlog drains in order

TST-13  Multi-beacon edge cases

- Block one beacon mid-test (bag of water)

- Confirm fingerprint fallback handles gracefully

TST-14  Caregiver UX walkthrough

- Non-team participant attempts core flows

- Capture friction points and re-prioritise polish work

EV · Evaluation · 8 tasks

EV-01  Define evaluation protocol

- Test scenarios mapped to each success metric

- Roles, equipment list, data capture method

- Reviewed by team before execution

EV-02  Establish baseline (current care model)

- Caregiver response time without device (literature or simulated)

- Monitoring duration without device

EV-03  Measure response time with device

- Triggered events vs caregiver visibility latency

- Multiple trial runs for statistical confidence

EV-04  Measure monitoring duration

- Time spent actively checking on the patient before vs with device

- Self-report or observed

EV-05  False positive / negative analysis

- Per alert type: count and classify

- False negative rate flagged as critical metric in original plan

EV-06  Caregiver burnout indicators

- Short qualitative survey design

- Even a small-N pilot is valuable framing data

EV-07  Compile evaluation results

- Summary table of metrics vs targets

- Charts for the final report and presentation

EV-08  Map findings to Biodesign framework

- Reflect on identification → solution → prototype → evaluation arc

- Document where the framework helped and where it strained

REG · Regulatory & Privacy · 10 tasks

REG-01  TGA classification

- Identify likely class for the wearable

- Document evidence and reasoning

REG-02  ISO 13485 mapping

- Identify clauses applicable to a prototype-stage product

- Note gaps to be closed before commercial launch

REG-03  IEC 60601-1 considerations

- Electrical safety obligations for body-worn devices

- Document which sub-parts apply and which don't

REG-04  Australian Privacy Act compliance

- Health information classed as sensitive under APP

- Collection, use, storage, and disclosure documented

REG-05  GDPR mapping (international scenarios)

- Lawful basis for processing

- Data subject rights and how the system supports them

REG-06  Data retention policy

- How long sensor data persists in active storage

- Archival and deletion procedures

REG-07  Consent flow for caregivers

- Terms of service / privacy policy on signup

- Caregiver acknowledgement captured in audit log

REG-08  Patient consent surrogate

- Where the patient cannot consent, who consents on their behalf

- Document chain of authority for prototype scope

REG-09  Data breach response plan

- Notification procedure

- Containment and remediation steps

REG-10  Ethics approval for clinical pilot

- Identify the body that would review

- Outline what's needed before a real pilot can begin

DOC · Documentation & Deliverables · 12 tasks

DOC-01  Consolidated system architecture document

- Block diagram, sequence diagrams, data flow

- Living doc — updated through the build

DOC-02  Final Bill of Materials with costing

- Per-unit and per-build cost

- Source links, lead times

DOC-03  Firmware README & setup guide

- Build environment, flashing instructions, troubleshooting

- Useful for examiners reproducing the demo

DOC-04  Backend API contract docs

- Edge function inputs / outputs

- Schema diagrams

DOC-05  Predictive model V2 roadmap

- Why ML is deferred

- What V1 instrumentation collects to make V2 training feasible

DOC-06  Regulatory & privacy summary

- Compiles outputs from REG-01 through REG-10

- Section in the final report

DOC-07  Final project report — first draft

- All team sections wired together

- Internal review pass

DOC-08  Final report — incorporate evaluation results

- Charts and metrics from EV-07

- Discussion of gaps and future work

DOC-09  Biodesign reflection section

- How the framework shaped the project

- Honest reflection on what went well and what didn't

DOC-10  Final presentation deck

- Single unified deck (no version language)

- Brand-consistent slide styling

DOC-11  Demo script & rehearsal

- Step-by-step narration with timing

- Two dry runs minimum

DOC-12  Q&A preparation

- Anticipated questions per workstream

- Pre-written answers reviewed by the relevant lead

# Critical Path

If anything must move first, it's these:

- HW-02 / HW-03 — procurement. No components, no build.

- BE-01 / BE-02 — Supabase project + initial schema. Unblocks UI and ingestion in parallel.

- FW-03 / FW-04 — sensor drivers up and reading. Validates the hardware story end-to-end.

- MQ-02 — broker reachable. Without it, FW and BE can't integrate.

- UI-08 / UI-09 — dashboard shell + live sensor cards. First demo-able artefact.

# Stretch / V2 Deferrals

Explicitly out of scope for V1, but worth keeping warm:

- ML-driven adaptive alert thresholds (background training pipeline)

- AR / camera-based floor plan auto-mapping

- GPS-walked perimeter mapping

- Hub aggregation layer (proper edge hub per facility)

- Multi-tenant facility admin

- Native caregiver mobile app with push notifications

- Optional vision module on the wearable

- FHIR-shaped clinical data export